CARE OF ADDRESS

Care-Of Address (CoA) is a temporary IP address assigned to a mobile node (such as a smartphone or laptop) when it moves away from its home network into a "foreign" network.  
It acts as a local pointer that tells the internet where to send data so the device can maintain its connection while roaming.

Key Functions of CoA

- Identity vs. Location: The CoA separates a device's identity (Home Address) from its current location (Care-Of Address), allowing communication to continue without resetting connections.

- Tunnel Endpoint: It serves as the termination point of a network "tunnel" from the Home Agent. The Home Agent encapsulates packets and redirects them to the CoA.

- Transparency: The CoA is typically invisible to the external "Correspondent Node" communicating with the device; the outside world still addresses packets to the device's permanent Home Address.

Types of Care-Of Addresses

There are two primary methods for assigning a CoA:

- Foreign Agent Care-Of Address (FA-CoA):

- Source: Provided by a Foreign Agent (a router in the visited network).

- Mechanism: Multiple mobile nodes can share the same FA-CoA, which is usually the IP address of the Foreign Agent itself.

- Efficiency: This method conserves IP addresses because a unique public IP is not required for every roaming device

- Co-Located Care-Of Address (CCoA):

- Source: Assigned directly to the mobile node's own network interface via protocols like DHCP or PPP.

- Mechanism: The mobile node acts as its own tunnel endpoint and performs its own de-encapsulation.

- Autonomy: This allows a device to function in foreign networks even if they do not have a dedicated Foreign Agent deployed

Registration Process

When a mobile node enters a new network, it follows these steps to use its CoA:

- Agent Discovery: The node detects it has moved and identifies the local Foreign Agent or gets a new local address.

- Registration Request: The node informs its Home Agent of its new CoA (often through the Foreign Agent).

- Binding: The Home Agent creates a "mobility binding," mapping the permanent Home Address to the temporary CoA until the registration expires.